Control Interface Document

RocketPower Avionics System

Document ID: CID-RocketPower-Avionics

| Revision | Date | Description |
| --- | --- | --- |
| Rev 00 | 2025-12-28 | Initial release |

Prepared by: Anas Senouci

Project: RocketPower Flight Computer

Contents

Abstract	3

1. Introduction	3

1.1 Purpose	3

1.2 Scope	3

1.3 Definitions and Acronyms	3

2. System Overview	3

2.1 System Description	3

2.2 High-Level Architecture	4

3. Power Interfaces	4

3.1 Battery Interface	4

3.2 Regulated Power Rails	4

4. Sensor Interfaces	5

4.1 IMU Interface	5

5. Communication Interfaces	5

5.1 UART Interfaces	5

5.2 SPI Interfaces	5

6. Pyrotechnic Control Interfaces	5

6.1 Pyrotechnic Channel Overview	5

6.2 Pyro Channel Control Logic	6

6.3 Pyro Firing Path	6

6.4 Continuity Sensing Interface	6

7. Safety and Arming Interfaces	6

7.1 Hardware Arming Switch	6

7.2 Default Safe State	7

8. Functional Behavior	7

8.1 Normal Operation	7

8.2 Pyrotechnic Activation	7

9. Constraints and Assumptions	7

10. Verification and Validation	7

Final note	8

# Abstract

This document defines the control, electrical, and logical interfaces of the RocketPower avionics system. The purpose of this Control Interface Document (CID) is to formally specify how subsystems interact, including power distribution, sensor interfaces, communication links, and pyrotechnic control channels. This document is intended to support system integration, verification, and safety review activities.

# 1. Introduction

## 1.1 Purpose

The purpose of this document is to define all control interfaces associated with the RocketPower avionics system. This includes logical control signals, power interfaces, communication buses, and pyrotechnic initiation interfaces.

## 1.2 Scope

This CID covers:

- Electrical power interfaces

- Sensor interfaces

- Communication interfaces

- Pyrotechnic channel control interfaces

- Safety and arming interfaces

Mechanical interfaces and firmware implementation details are outside the scope of this document unless directly related to control behavior.

## 1.3 Definitions and Acronyms

| Term | Definition |
| --- | --- |
| CID | Control Interface Document |
| MCU | Microcontroller Unit |
| IMU | Inertial Measurement Unit |
| GPIO | General Purpose Input/Output |
| PYRO | Pyrotechnic output channel |
| VBAT | Battery voltage rail |
| GND | Electrical ground reference |

# 2. System Overview

## 2.1 System Description

The RocketPower avionics system is a centralized flight computer responsible for:

- Sensor data acquisition

- Flight state estimation

- Data logging

- Telemetry communication

- Pyrotechnic event control

The system is powered by a rechargeable 2s battery (7.4v) and interfaces with external sensors, actuators, and pyrotechnic devices.

## 2.2 High-Level Architecture

At a high level, the system consists of a MCU based off the Teensy 4.1 architecture.

# 3. Power Interfaces

## 3.1 Battery Interface

| Parameter | Description |
| --- | --- |
| Input Voltage | Nominal 2S battery (7.4v) |
| Connector Type | Screw terminal |
| Function | Primary system power input |

## 3.2 Regulated Power Rails

| Rail | Voltage | Usage |
| --- | --- | --- |
| 3V3 | 3.3 V | MCU, sensors, logic |
| +7.5V | Battery voltage | Pyrotechnic firing power and servos |

## 3.3 Current consumption 7.4v rail

| Component | Qty | Max curr/unit | Max curr total | Note |
| --- | --- | --- | --- | --- |
| Blue Bird BMS-101HV servo | 4 | 0.5 A | 2 A | Current stall has been measured through RD6012 power supply, no official documentation. |
| RunCam Split 4-25 | 1 | 450 mA | 450 mA | At 5v |
| E-match 89866 | 3 | 1 A | 1 A | Used once at a time |
| 3.3v rail | 1 | 388.295 mA | 388.295 mA |  |
| Total (50% Margin of Safety) |  |  | 5.75 A | Trace width: 1 oz/ft2 = 225mil 2 oz/ft2 = 113mil 3 oz/ft2 = 75mil 4 oz/ft2 = 56mil |

*Assuming a 20 degrees Celsius temperature rise and inner power layer

## 3.4 Current consumption 3.3v rail

| Component | Qty | Max curr/unit | Max curr total | Note |
| --- | --- | --- | --- | --- |
| XBee XB24CAPIT-001 | 1 | 33 mA | 33 mA |  |
| u-blox ZOE-M8Q | 1 | 67 mA | 67 mA |  |
| BNO085 | 1 | 30 mA | 30 mA |  |
| BMP390 | 1 | 0.73 mA | 0.73 mA |  |
| INA260 | 1 | 0.42 mA | 0.42 mA |  |
| ADXL375 | 1 | 0.145 mA | 0.145 mA |  |
| microSD card | 1 | 100 mA | 100 mA |  |
| QSPI flash | 1 | 20 mA | 20 mA |  |
| PSRAM | 1 | 7 mA | 7 mA |  |
| MCU RT1062 | 1 | 100 mA | 100 mA |  |
| LED HSMF-C169 | 6 | 5 mA | 30 mA |  |
| Total (50% Margin of Safety) |  |  | 388.295 mA *1.5 = 0.6 A |  |

# 4. Sensor Interfaces

## 4.1 IMU Interface

| Signal | Type | Description |
| --- | --- | --- |
| SDA | I2C | Data line |
| SCL | I2C | Clock line |
| 3V3 | Power | Sensor supply |
| GND | Power | Ground reference |

4.2 Environmental Sensors

Environmental sensors interface via I2C and share the same electrical characteristics as the IMU interface.

# 5. Communication Interfaces

## 5.1 UART Interfaces

UART interfaces are used for telemetry, GPS, and optional peripherals.

| Signal | Direction | Description |
| --- | --- | --- |
| TX | Output | MCU transmit |
| RX | Input | MCU receive |
| GND | Reference | Signal ground |
| 3V3 | Power | Logic supply |

UART signals use 3.3 V logic levels.

## 5.2 SPI Interfaces

SPI interfaces may be used for data logging devices such as microSD.

| Signal | Description |
| --- | --- |
| MOSI | Master out, slave in |
| MISO | Master in, slave out |
| SCK | Serial clock |
| CS | Chip select |

# 6. Pyrotechnic Control Interfaces

## 6.1 Pyrotechnic Channel Overview

Each pyrotechnic channel consists of:

- Control logic

- Low-side switching MOSFET

- Dedicated firing output

- Continuity sensing path

The design separates logic control from high-current firing paths.

## 6.2 Pyro Channel Control Logic

The pyro channel control logic:

- Receives a GPIO command from the MCU

- Drives the MOSFET gate

- Includes gate pulldown for default-off behavior

- Does not carry firing current

## 6.3 Pyro Firing Path

| Element | Description |
| --- | --- |
| Power Source | PYRO_V+ |
| Switch | Low-side MOSFET |
| Load | E-match / igniter |
| Return | System ground |

No intentional resistance is placed in the firing current path.

## 6.4 Continuity Sensing Interface

Continuity sensing is implemented via a high-value resistor path to limit current below the igniter no-fire threshold.

| Parameter | Value |
| --- | --- |
| Maximum test current | < 40 mA |
| Resistor range | kΩ order |
| Purpose | Detect igniter presence without ignition |

# 7. Safety and Arming Interfaces

## 7.1 Hardware Arming Switch

A hardware arming switch physically interrupts the PYRO_V+ rail.

| State | Behavior |
| --- | --- |
| Disarmed | Pyrotechnic channels unpowered |
| Armed | Pyrotechnic channels enabled |

This provides a hardware-level inhibit independent of software state.

## 7.2 Default Safe State

On power-up or MCU reset:

- All pyro control GPIOs default low

- MOSFET gates are pulled down

- Pyrotechnic firing is impossible unless explicitly armed

# 8. Functional Behavior

## 8.1 Normal Operation

During normal operation:

- Sensors are continuously sampled

- Data is logged and/or transmitted

- Pyrotechnic channels remain disarmed unless commanded

## 8.2 Pyrotechnic Activation

Pyrotechnic activation requires:

- Hardware arming switch enabled

- Valid software command

- Correct system state

# 9. Constraints and Assumptions

- Battery voltage may vary under load

- Pyrotechnic firing currents are transient and high

- Sensor accuracy depends on power integrity and layout

- All interfaces assume common ground reference

# 10. Verification and Validation

Interface verification shall include:

- Electrical continuity tests

- Voltage level verification

- Current capability testing

- Functional activation tests with inert loads

# Final note

This is a proper CID, not a class report.
You can now:

- paste this into Word

- add diagrams

- cross-reference schematics

- submit it or review it like a real avionics document

If you want, next I can:

- tailor this exactly to your schematic net names

- add tables for each connector

- split it into avionics vs pyro annexes

- convert it into a requirements-traceable CID