Control Interface Document

RocketPower Avionics System

Document ID: CID-RocketPower-Avionics
Prepared by: Anas Senouci
Project: RocketPower Flight Computer

Revision History

| Revision | Date | Description |
| --- | --- | --- |
| Rev 00 | 2025-12-28 | Initial skeleton, not yet checked against schematics/firmware. |
| Rev 01 | 2026-08-09 | Verified against PCB/main schematics and FlightComputer/src. Added command/telemetry packet spec (5.5), actuation interfaces (6), backup-deployment safety path (8.4), GPIO appendix. Corrected: SD is SDIO not SPI (5.3); pyro path has an intentional 7ohm current-limit resistor (7.3); radio is LoRa E22-900T22D, not the legacy XBee reference (5.1). |
| Rev 02 | 2026-08-10 | Added Section 13 (mass/CG/CP tracking). Trimmed document-wide for length. |

# Abstract

Defines the control, electrical, and logical interfaces of the RocketPower avionics system -- a custom flight computer (bare NXP MIMXRT1062, Teensy 4.1-compatible), its ground station, and the KiCad PCB connecting them to sensors, radio, pyro channels, and actuated fins -- so the system can be understood, verified, or reproduced without reading firmware/schematic source directly.

# 1. Introduction

## 1.1 Purpose

Defines all control interfaces of the RocketPower avionics system: logical control signals, power interfaces, communication protocols, actuation interfaces, and pyrotechnic initiation interfaces.

## 1.2 Scope

Electrical power interfaces

Sensor interfaces

Communication interfaces, including the full ground<->flight binary packet protocol

Actuation interfaces (fins, camera, status indicators)

Pyrotechnic channel control interfaces

Safety and arming interfaces

Flight state machine behavior as it affects control interfaces

Mechanical design and firmware internals not tied to control behavior are out of scope.

## 1.3 Definitions and Acronyms

| Term | Definition |
| --- | --- |
| CID | Control Interface Document |
| MCU | Microcontroller Unit (NXP MIMXRT1062) |
| FC | Flight Computer |
| GCS | Ground Control Station |
| IMU | Inertial Measurement Unit |
| GPIO | General Purpose Input/Output |
| PWM | Pulse-Width Modulation |
| I2C | Inter-Integrated Circuit bus |
| SDIO | Native 4-bit microSD bus |
| UART | Serial link |
| RSSI | Received Signal Strength Indicator |
| PYRO | Pyrotechnic output channel |
| +BATT | Raw battery rail, upstream of the main fuse |
| +7.5V | Fused, monitored, unregulated rail feeding pyro and servos |
| RBF | Remove-Before-Flight hardware link |
| LoRa | Modulation used by the E22-900T22D telemetry radio |

# 2. System Overview

## 2.1 System Description

The RocketPower flight computer handles: sensor acquisition, flight-state estimation, SD logging, ground telemetry/commands, pyrotechnic control, and optional closed-loop fin actuation. Powered by a 2S LiPo (7.4 V nominal).

## 2.2 High-Level Architecture

Bare NXP MIMXRT1062DVL6B (Teensy 4.1 silicon) on a custom PCB, with an MKL02Z32VFG4 coprocessor for USB/bootloader, flashed via PlatformIO/Teensy toolchain.

Power: +BATT -> fuse -> INA260 shunt -> TPS62111RSAR buck (3.3V) + unregulated +7.5V (pyro/servos)

Sensing: 5 I2C sensors across 3 buses (Section 4)

Storage: microSD over native SDIO (5.3)

Comms: LoRa UART telemetry link, UART camera link, USB-C (5)

Actuation: 3 pyro channels, 4 fin servos (6-7)

Indication: 4 status LEDs + 2 hardwired indicators (6.3)

Safety: hardware RBF link, software arm/disarm, independent backup-deployment monitor (8)

| State | Meaning |
| --- | --- |
| IDLE | Powered on, waiting for ARM |
| ARMED | Pyro armed; liftoff detection active |
| POWERED_ASCENT | Motor burning |
| COAST | Burned out, coasting to apogee |
| APOGEE | Apogee confirmed; CH2 fires |
| DESCENT | Descending under chute |
| LANDED | Altitude stable; flight over |

# 3. Power Interfaces

## 3.1 Battery Interface

| Parameter | Description |
| --- | --- |
| Input Voltage | 2S LiPo, 7.4 V nominal (net: +BATT) |
| Connector | 2-pos screw terminal (Screw_Terminal_01x02, "BATT") |
| Protection | 6 A fuse (F1) to the downstream +7.5V rail |
| Bulk Storage | 470 uF electrolytic (C41) on +7.5V |

## 3.2 Regulated Power Rails

| Rail | Voltage | Source | Usage |
| --- | --- | --- | --- |
| 3V3 | 3.3 V regulated | TPS62111RSAR buck, from +BATT | MCU, sensors, radio logic, LEDs |
| +7.5V | Battery voltage, fused/unregulated | +BATT through F1 and INA260 shunt | Pyro fire power, servos, camera |

INA260's shunt sits between +BATT and +7.5V, so telemetry voltage/current reflect that fused rail, not raw battery current.

## 3.3 Current Budget -- +7.5V Rail

| Component | Qty | Max/unit | Max total | Note |
| --- | --- | --- | --- | --- |
| Blue Bird BMS-101HV servo | 4 | 0.5 A | 2 A | Bench-measured stall current |
| RunCam Split 4-25 | 1 | 450 mA | 450 mA | At 5 V -- see Section 12 |
| E-match (89866) | 3 | 1 A | 1 A | Used one at a time |
| 3.3V rail (reflected) | 1 | 388.295 mA | 388.295 mA |  |
| Total (50% margin) |  |  | 5.75 A | Below the 6 A fuse rating |

## 3.4 Current Budget -- 3.3V Rail

| Component | Qty | Max/unit | Max total | Note |
| --- | --- | --- | --- | --- |
| LoRa E22-900T22D radio | 1 | TBD | TBD | See Section 12 -- not yet re-measured for this module |
| u-blox ZOE-M8Q GPS | 1 | 67 mA | 67 mA |  |
| BNO085 IMU | 1 | 30 mA | 30 mA |  |
| BMP390 barometer | 1 | 0.73 mA | 0.73 mA |  |
| INA260 power monitor | 1 | 0.42 mA | 0.42 mA |  |
| ADXL375 high-g accel | 1 | 0.145 mA | 0.145 mA |  |
| microSD card | 1 | 100 mA | 100 mA |  |
| QSPI flash | 1 | 20 mA | 20 mA |  |
| PSRAM | 1 | 7 mA | 7 mA |  |
| MCU | 1 | 100 mA | 100 mA |  |
| Status LEDs | 6 | 5 mA | 30 mA |  |
| Total (excl. radio, 50% margin) |  |  | ~0.6 A | Excludes radio TBD above |

# 4. Sensor Interfaces

| Bus | MCU peripheral | Pins | Devices |
| --- | --- | --- | --- |
| Wire | LPI2C1 | SDA 18 / SCL 19 | BNO085 IMU |
| Wire1 | LPI2C3 | SDA 17 / SCL 16 | BMP390, ZOE-M8Q GPS, INA260 |
| Wire2 | LPI2C4 | SDA 25 / SCL 24 | ADXL375 |

## 4.1 IMU -- BNO085

Wire, addr 0x4A (SA0 low)

BNO085_RST hardware reset line to MCU

Outputs orientation quaternion + gyro rates

## 4.2 Barometer -- BMP390

Wire1, addr 0x77 (SDO high)

Relative altitude zeroed at pad (CALIBRATE_BARO); vertical velocity via complementary filter (alpha=0.98) with IMU

## 4.3 High-g Accelerometer -- ADXL375

Wire2, addr 0x1D, range +-200 g

Drives liftoff (>2.5 g / 100 ms) and burnout (<1.0 g) detection

## 4.4 GPS -- u-blox ZOE-M8Q

Wire1, fixed addr 0x42

External antenna via u.FL/MHF1 connector

## 4.5 Power Monitor -- INA260

Wire1, addr 0x40 (A0=A1=GND)

Shunt between +BATT and +7.5V

## 4.6 Common I2C Signals

| Signal | Description |
| --- | --- |
| SDA / SCL | Data / clock |
| 3V3 / GND | Sensor supply / ground reference |

# 5. Communication Interfaces

## 5.1 Telemetry Radio Link

| Parameter | Value |
| --- | --- |
| Module | LoRa E22-900T22D, Mode 0 (transparent UART). Supersedes an earlier XBee reference; see Section 12. |
| MCU interface | Serial1 (Lora_TX/Lora_RX) |
| Connector | 4-pin JST-PH ("TELEMETRY"): 3V3, GND, TX, RX |
| Baud | 9600 fixed |
| Control lines | M0/M1/AUX not connected to MCU (hardwired Mode 0) |
| RF channel | Channel 0 = 900.125 MHz (+1 MHz/step) |
| Throughput | ~2.4 kbps air rate, half-duplex -> 1 Hz telemetry (TELEMETRY_INTERVAL_MS) to leave room for uplinked commands |

## 5.2 Camera Link

| Parameter | Value |
| --- | --- |
| Device | RunCam Split 4-25 compatible |
| MCU interface | Serial7 (pins 28/29) |
| Connector | 4-pin JST-PH ("CAMERA") |
| Baud | 115200 |
| Power | +7.5V rail -- verify against camera's rated input (Section 12) |
| Protocol | Toggle-only, no ack; cam_recording in telemetry is FC's belief |

## 5.3 Storage -- microSD

| Parameter | Value |
| --- | --- |
| Connector | Hirose DM3D-SF microSD socket |
| Interface | Native 4-bit SDIO (BUILTIN_SDCARD): CLK, CMD, DAT0-3. Not SPI. |
| Card detect | None -- presence inferred from SD.begin()/write attempts |
| Log format | FLTXXXXX.BIN: ASCII CSV header + binary FlightData records at 100 Hz |
| Re-probe policy | Not polled while idle (SD.begin() is slow with no card and caused telemetry stalls); re-checked at boot, on SD_START_RECORDING, or on a write failure |

## 5.4 USB Interface

| Parameter | Value |
| --- | --- |
| Connector | USB-C (USB2.0) |
| Functions | Firmware upload (Teensy bootloader); 115200 debug console; optional binary telemetry mirror |
| Telemetry mirror | USB_SERIAL_BINARY_MIRROR=1 (default): raw packet mirrored at 20 Hz (USB_TELEMETRY_INTERVAL_MS=50), independent of the LoRa rate |

## 5.5 Command / Telemetry Packet Protocol

Binary, little-endian, magic-prefixed, checksummed (16-bit sum of preceding bytes). Authoritative for both GroundStation/core/packet_*.py and FlightComputer/src/telemetry/Packet.h.

### 5.5.1 TelemetryPacket (FC -> Ground), 52 bytes, magic 0xAA 0x55

| Offset | Size | Type | Field | Scale / Notes |
| --- | --- | --- | --- | --- |
| 0 | 2 | uint8[2] | magic | 0xAA, 0x55 |
| 2 | 2 | uint16 | seq | Rolling sequence number |
| 4 | 4 | uint32 | timestamp_ms | millis() since boot |
| 8 | 1 | uint8 | state | FlightState (9.2) |
| 9 | 4 | float | lat | Degrees |
| 13 | 4 | float | lon | Degrees |
| 17 | 2 | int16 | gps_alt_dm | /10 = meters |
| 19 | 1 | uint8 | gps_sats |  |
| 20 | 1 | uint8 | gps_fix |  |
| 21 | 2 | int16 | baro_alt_dm | /10 = meters, relative to pad |
| 23 | 2 | int16 | vert_vel_cms | /100 = m/s, + = up |
| 25 | 2 | int16 | accel_x_cg | /100 = g |
| 27 | 2 | int16 | accel_y_cg | /100 = g |
| 29 | 2 | int16 | accel_z_cg | /100 = g |
| 31 | 2 | int16 | quat_w_i16 | /32767 |
| 33 | 2 | int16 | quat_x_i16 | /32767 |
| 35 | 2 | int16 | quat_y_i16 | /32767 |
| 37 | 2 | int16 | quat_z_i16 | /32767 |
| 39 | 2 | int16 | voltage_cv | /100 = V |
| 41 | 2 | int16 | current_ma |  |
| 43 | 1 | int8 | rssi | Stub: getRSSI() returns -1, unused in Mode 0 |
| 44 | 3 | uint8[3] | pyro_cont[0..2] | 1=OK 0=open, CH1/CH2/CH3 |
| 47 | 1 | uint8 | cam_recording | FC's belief, no camera ack |
| 48 | 1 | uint8 | system_status | Bitfield, 5.5.5 |
| 49 | 1 | uint8 | attitude_status | Bitfield, 5.5.6 |
| 50 | 2 | uint16 | checksum | Sum of bytes 0..49 |

### 5.5.2 CommandPacket (Ground -> FC), 7 bytes, magic 0xBB 0x44

| Offset | Size | Type | Field | Notes |
| --- | --- | --- | --- | --- |
| 0 | 2 | uint8[2] | magic | 0xBB, 0x44 |
| 2 | 1 | uint8 | type | CommandType, 5.5.4 |
| 3 | 1 | uint8 | param | Channel for FIRE_PYRO/SERVO_*; else 0 |
| 4 | 1 | uint8 | seq | Ground-assigned, echoed in ack, used for dedup |
| 5 | 2 | uint16 | checksum | Sum of bytes 0..4 |

### 5.5.3 AckPacket (FC -> Ground), 6 bytes, magic 0xAC 0x4B

| Offset | Size | Type | Field | Notes |
| --- | --- | --- | --- | --- |
| 0 | 2 | uint8[2] | magic | 0xAC, 0x4B |
| 2 | 1 | uint8 | cmdSeq | Echoes CommandPacket.seq |
| 3 | 1 | uint8 | cmdType | Echoes CommandPacket.type |
| 4 | 2 | uint16 | checksum | Sum of bytes 0..3 |

Sent on receipt, before execution -- so a slow command (e.g. SERVO_PREFLIGHT) still gets acked promptly.

### 5.5.4 CommandType Enumeration

| Value | Name | Param | Effect |
| --- | --- | --- | --- |
| 0x01 | ARM | - | Arms pyro; enters ARMED; recalibrates baro; starts camera |
| 0x02 | DISARM | - | Disarms pyro; forces fire GPIOs low; clears attitude flags |
| 0x03 | FIRE_PYRO | 1-3 | Fires channel if armed (Section 7-8) |
| 0x04 | PING | - | Liveness check |
| 0x05 | CALIBRATE_BARO | - | Re-zeros altitude at current level |
| 0x06 | SERVO_TEST | fin 1-4 | Sweep: center -> min -> max -> center |
| 0x07 | CAM_TOGGLE | - | Toggles camera recording |
| 0x09 | SERVO_NUDGE_POS | fin 1-4 | +1 trim step (unsaved) |
| 0x0A | SERVO_NUDGE_NEG | fin 1-4 | -1 trim step (unsaved) |
| 0x0B | SERVO_SAVE_CAL | - | Persists live position as new trim |
| 0x0C | SERVO_CENTER_ALL | - | All 4 fins to raw center (unsaved) |
| 0x0D | SERVO_PREFLIGHT | - | Blocking ~6s 4-channel choreography |
| 0x0E | SD_START_RECORDING | - | Opens a new log file |
| 0x0F | SD_STOP_RECORDING | - | Flushes and closes the log file |
| 0x10/0x11 | ATTITUDE_CONTROL_EN/DIS | - | Real in-flight fin control (9.4) |
| 0x12/0x13 | ATTITUDE_DEMO_EN/DIS | - | Ground bench-demo fin control (9.4) |

All commands are ack-confirmed, sequence-numbered, and safely auto-retried by the ground station; the firmware dedupes per command type.

### 5.5.5 system_status Bitfield (byte 48)

| Bit | Name | Meaning |
| --- | --- | --- |
| 0-4 | SENSOR_HEALTH_(IMU/BARO/ACCEL/GPS/POWER)_OK | Read successfully within last 500 ms |
| 5 | SD_STATUS_PRESENT | Card answered last presence check |
| 6 | SD_STATUS_RECORDING | Log file currently open |

### 5.5.6 attitude_status Bitfield (byte 49)

| Bit | Name | Meaning |
| --- | --- | --- |
| 0 | ATTITUDE_STATUS_CONTROL_ON | Real in-flight control commanded ON |
| 1 | ATTITUDE_STATUS_DEMO_ON | Ground bench-demo commanded ON |

# 6. Actuation Interfaces

## 6.1 Fin Servo Interface

| Parameter | Value |
| --- | --- |
| Servo | Blue Bird BMS-101HV (6.0-8.4 V), 4 channels |
| Connector | 3-pin JST-VH per channel |
| Power | +7.5V rail, shared with pyro |
| Control | PWM via Arduino Servo library |
| Center pulse | 1520 us (datasheet neutral) |
| Software limit | +-25 deg/side (rated travel +-45 deg) |
| Trim | Per-channel EEPROM offset, set via ground nudge commands, applied at boot |
| Boot sequencing | 400 ms stagger per channel to limit inrush on the shared rail |
| Commanded by | Ground bench commands, or the optional AttitudeController (9.4) |

## 6.2 Camera Control

See 5.2. Toggle-only; no hardware ack.

## 6.3 Status Indicator Interface

| Designator | Function | Behavior |
| --- | --- | --- |
| D1 | 3.3V LED | Hardwired power-good, not GPIO-driven |
| D2 | BOOT LED | Driven by MKL02 coprocessor, not firmware |
| D3 | GPS LED | Solid = GPS healthy + fix; flashing ~2 Hz = not |
| D4 | Altimeter LED | Solid = baro healthy; flashing = not |
| D5 | 200G ACC LED | Solid = accel healthy; flashing = not |
| D6 | IMU LED | Solid = IMU healthy; flashing = not |

Tactile reset switch (KMR241GLFS) wired directly to MCU reset.

# 7. Pyrotechnic Control Interfaces

## 7.1 Channel Overview

| Channel | Function | Fire pin | Continuity pin |
| --- | --- | --- | --- |
| CH1 | Motor ignition (ground-commanded) | 2 | 40 |
| CH2 | Parachute -- auto-fires at APOGEE | 3 | 41 |
| CH3 | Backup -- manual anytime after ARM, or auto by BackupDeploy (8.4) | 4 | 39 |

## 7.2 Pyro Channel Control Logic

3x DMG3406L N-channel MOSFETs (Q1/Q2/Q3), low-side switches, one per channel

3.3V GPIO drives the gate directly -- HIGH fires, LOW is off

10k-ohm gate pulldown per channel for default-OFF

Firmware forces all fire GPIOs LOW at begin() and disarm()

## 7.3 Pyro Firing Path

7-ohm resistor in series with each e-match, sized for ~1 A from the ~8V rail (not an unlimited short)

Fire pulse: 500 ms (PYRO_FIRE_DURATION_MS)

Output via an 8-position screw terminal (Phoenix 1725711); confirm pin-to-channel mapping against main.kicad_pcb (Section 12)

## 7.4 Continuity Sensing

7.32k-ohm series resistor per channel into an analog MCU pin, ~0.35 mA -- well under e-match no-fire thresholds

Threshold: analogRead() > 512 = continuity OK

Sampled every loop, reported live (pyro_cont[0..2])

PyroController::fire() does not check continuity -- it is display-only, not a firmware interlock. See Section 12.

# 8. Safety and Arming Interfaces

## 8.1 Hardware Arming (Remove-Before-Flight)

A 2-pin removable link in the pyro section (schematic note: "Remove before flight switch") provides a hardware inhibit independent of firmware. Exact net not traced to copper here -- confirm against main.kicad_pcb (Section 12).

## 8.2 Software Arm/Disarm

ARM (0x01) / DISARM (0x02) drive both the state machine's ARMED state and PyroController's armed flag

Boot default: disarmed, all fire GPIOs LOW

ARM also recalibrates baro at the pad and starts the camera

DISARM (or reaching LANDED) forces fire GPIOs low and clears attitude flags

## 8.3 Default Safe State

All fire GPIOs default LOW at boot (backed by hardware pulldowns); PyroController boots disarmed.

## 8.4 Independent Backup Deployment

State-machine state and the armed flag are RAM-only and don't survive an MCU reset, so a mid-flight reset could otherwise strand the vehicle in IDLE with no way back to APOGEE. BackupDeploy runs every loop, unconditionally, and independently re-derives the same evidence chain from raw sensor data: sustained boost accel -> >=50 m real altitude gain -> velocity zero-crossing -> 2 s delay. If satisfied, it fires CH3 via fireBackupUnconditional(), bypassing the armed check. Fires at most once per boot.

## 8.5 Pyro Activation Preconditions

fire() requires only: (1) channel is software-armed, (2) valid channel number. No flight-state gate, no continuity gate. CH2 auto-fires once at APOGEE; any channel can be fired manually after ARM regardless of state (see FLIGHT_PROCEDURE.md Section 5). The RBF link (8.1) is the only gate outside firmware.

# 9. Functional Behavior

## 9.1 Normal Operation

Sensors sampled/fused each loop (~100 Hz)

Logged to SD at 100 Hz, telemetered at 1 Hz (LoRa) / 20 Hz (USB)

Pyro stays disarmed until ground ARM

FC flies autonomously; ground station is visibility + manual override only

## 9.2 Flight State Machine

| Transition | Trigger |
| --- | --- |
| IDLE -> ARMED | Ground ARM |
| ARMED -> POWERED_ASCENT | Accel > 2.5 g for >= 100 ms |
| POWERED_ASCENT -> COAST | Accel drops below 1.0 g |
| COAST -> APOGEE | Vertical velocity + to -, confirmed over 200 ms |
| APOGEE -> DESCENT | Immediate; CH2 fires |
| DESCENT -> LANDED | Altitude stable +-2 m for 10 s |
| -> IDLE | Ground DISARM |

Camera stops on LANDED; log flushes on every transition, closes on LANDED; telemetry keeps transmitting regardless of ground link state.

## 9.3 Pyrotechnic Activation Logic

See Section 8.5.

## 9.4 Active Attitude Control (Optional, Off by Default)

Closed-loop fin control (AttitudeController); both enable flags default OFF and reset OFF on DISARM.

PD controller per axis: ANGLE term (drift from a reference orientation latched at engage) + RATE term (damping)

ATTITUDE_CONTROL_EN/DIS: real in-flight, POWERED_ASCENT/COAST only. ATTITUDE_DEMO_EN/DIS: ground bench test, ARMED only

Fin layout: CH1=S, CH2=E, CH3=N, CH4=W. Roll = all 4 fins; pitch = N/S diff; yaw = E/W diff

Gyro/quaternion axis-to-physical-rotation mapping is an UNVERIFIED placeholder -- must be bench-verified before real-flight use

# 10. Constraints and Assumptions

+7.5V rail is unregulated and tracks battery charge

7-ohm resistor and 6A fuse bound, but don't eliminate, pyro firing current risk

Flight state and armed flag are RAM-only -- why BackupDeploy (8.4) exists

LoRa link is half-duplex/airtime-limited; >1 Hz starves command reception

SD presence isn't polled while idle -- a card inserted without a ground command goes unnoticed

IMU axis mapping (9.4) is unverified -- not safe for real flight as-is

Servo rail is shared, unregulated, and competes with pyro headroom

# 11. Verification and Validation

Pyro continuity test per channel (ground station panel or bench meter)

Voltage/current checks on 3.3V and +7.5V rails against Section 3 budgets

Re-measure LoRa module current draw (Section 12)

Inert-load pyro fire tests + full SERVO_PREFLIGHT before closing the airframe

Confirm packet_decoder.py/packet_encoder.py match Packet.h after any protocol change

FLIGHT_PROCEDURE.md is the authoritative pre-flight checklist; this CID defines the interfaces it exercises.

# 12. Open Items

3.3V budget (3.4) still needs the LoRa module's real current draw measured (TX especially)

RBF link's exact interrupted net (8.1) not traced to PCB copper

8-position e-match terminal block pin-to-channel mapping (7.3) not enumerated -- check main.kicad_pcb

Camera supply voltage (+7.5V, 5.2) not confirmed against its rated input

IMU axis mapping (9.4) unverified -- required before real-flight ATTITUDE_CONTROL_ENABLE

fire() doesn't gate on continuity (7.4/8.5) -- confirm this is intended before treating it as a go/no-go

rssi field (5.5.1) is a stub, always -1

# 13. Physical and Simulation Properties

Mass/CG/CP tracking for stability and simulation. Values here change with build/motor configuration -- append new rows rather than overwriting.

## 13.1 Definitions

| Term | Definition |
| --- | --- |
| CG | Balance point of the rocket's mass; differs no-motor vs. loaded vs. burnout |
| CP | Point where net aerodynamic force acts; must sit aft of CG for stability |
| Stability margin | (CP - CG) / body diameter, in calibers; ~1-2 is a common target |
| Moment of inertia (I) | Resistance to angular acceleration about an axis; pitch/yaw about the CG here |
| Radius of gyration (k) | Mass-independent length scale where I = m * k^2 |

## 13.2 Methods

| Property | Method | Status |
| --- | --- | --- |
| CG (no motor / loaded / burnout) | Balance the assembled airframe on a finger/knife-edge; repeat per motor | Loaded recorded, 13.3; no-motor/burnout pending |
| CP | CAD planform-centroid ("flat-plate") method, 13.2.1 | Two estimates recorded (disagree) -- see 13.5, 13.3 |
| Total mass | Bench scale | Loaded + no-motor recorded, 13.3; propellant/burnout pending, 13.4 |
| Moments of inertia | Bifilar pendulum (two-string torsional), measured on the assembled rocket -- 13.2.2 | Measured, pitch/yaw, loaded config only -- 13.3 |

### 13.2.1 CP Method: CAD Planform Centroid

Side-profile silhouette sketched in Fusion 360, extruded to a uniform 1mm thickness, assigned a placeholder material (Steel -- only to give Fusion a density to compute a centroid), then read Center of Mass. Uniform thickness/density makes that centroid equal to the 2D silhouette's area centroid -- the classic "cardboard cutout" CP approximation. First-order only: ignores nose/fin lift-curve-slope differences and Mach effects. Cross-check against a Barrowman calculation or OpenRocket/RASAero before finalizing stability margin.

### 13.2.2 MOI Method: Bifilar Pendulum

Strings attached at 21 cm and 57 cm from the nose tip (CG +-18 cm), string separation d = 0.36 m; string length L = 1.70 m from a rigid overhead support, low-stretch string, girth hitches at attachment points; rocket hung level. Twisting a few degrees about the vertical axis and timing the torsional oscillation gives the moment of inertia -- the standard hobbyist bifilar-pendulum method. Measured LOADED (743 g, live motor installed). Raw data: 10 oscillations timed at 31 s -> T = 3.1 s. Formula: I = m*g*T^2*d^2/(16*pi^2*L); radius of gyration k^2 = g*T^2*d^2/(16*pi^2*L).

With g = 9.81 m/s^2: k^2 = 0.0455 m^2, k = 0.213 m, I_pitch = 0.743 kg * 0.0455 m^2 = 0.0338 kg*m^2 (loaded). Validation: a uniform 75 cm rod has k = L/sqrt(12) = 0.217 m; the measured 0.213 m sits just below, consistent with mass slightly concentrated toward the center (motor aft, nose weight forward). Assumes small-angle twist, equal/parallel strings, and negligible string-attachment offset from the centerline.

## 13.3 Recorded Values

| Property | Value | Method | Date | Notes |
| --- | --- | --- | --- | --- |
| Total length | 750 mm (75.0 cm) | Direct measurement | 2026-09-18 | Nose tip to tail |
| Body diameter | 66 mm (6.6 cm) | Direct measurement | 2026-09-18 | Reference diameter for aero coefficients and caliber calcs |
| CP, from nose tip | 361.006 mm (36.1 cm) | Fusion 360 planform projection (13.2.1); origin measured 656.139 mm from nose, nose-to-tail = 640.678 mm | 2026-08-10 |  |
| CP, from tail | 279.672 mm (28.0 cm) | Same measurement, opposite reference | 2026-08-10 |  |
| CP, from nose tip (OpenRocket) | 478 mm (47.8 cm) | OpenRocket simulation (Barrowman method) | 2026-09-18 | Cross-check per Open Items (13.5); disagrees with the CAD flat-plate estimate above by ~11.7 cm. Stability margin below uses this value. |
| CG (no motor) | NOT YET MEASURED | Balance test | - | Needed before computing stability margin |
| CG (loaded) | 390 mm (39.0 cm) | Balance test, per motor | 2026-09-18 | Motor installed (liftoff mass config) |
| Total mass (loaded) | 743 g (0.743 kg) | Bench scale | 2026-09-18 | Liftoff mass, live motor installed. The 354.106 g Fusion figure is the flat proxy body, not this value. |
| Mass, no motor | 656 g (0.656 kg) | Bench scale | 2026-09-18 | BENCH REFERENCE ONLY, not a flight state -- the rocket never flies without a motor |
| Motor mass (propellant + casing) | 87 g | Computed (loaded - no-motor) | 2026-09-18 | Includes the spent casing -- NOT propellant mass alone. Motor is single-use; casing stays onboard after burnout. |
| Propellant mass | TODO -- pending motor spec sheet | Motor manufacturer spec (thrustcurve.org) | - | Needed for the mass model, 13.4. Will be less than the 87 g motor mass above (casing excluded). |
| Mass at burnout | TODO = 743 g - propellant mass | Computed, pending propellant mass | - | Spent casing remains onboard -- burnout mass is NOT the 656 g no-motor reference |
| Radius of gyration, pitch/yaw (loaded) | 0.213 m | Bifilar pendulum, 13.2.2 | 2026-09-18 | Uniform-rod check predicts 0.217 m for this length; measured value sits just below |
| Moment of inertia, pitch/yaw (loaded) | 0.0338 kg*m^2 | Bifilar pendulum, I = m*k^2, 13.2.2 | 2026-09-18 | Held CONSTANT for the full simulated flight -- see 13.4 for justification and the required robustness test |
| Stability margin | 1.33 calibers | (CP-CG)/diameter, using OpenRocket CP | 2026-09-18 | = (47.8-39.0)/6.6, using the OpenRocket CP above. CAUTION: the CAD flat-plate CP (36.1 cm) instead gives a NEGATIVE margin (CP forward of CG) -- the two CP estimates disagree on stability itself, not just the number. See 13.5, unresolved. |

## 13.4 Modeling Assumptions and Limitations

The simulation holds pitch/yaw inertia FIXED at the loaded value (I = 0.0338 kg*m^2) for the entire flight rather than varying it over the burn. Deliberate simplification, not an oversight: only propellant leaves during flight (the casing stays onboard), so true inertia drops modestly from ignition to burnout, and the controller model is slightly optimistic near burnout as a result. REQUIRED before trusting this assumption: a robustness test perturbing pitch inertia by +-25% (deliberately conservative vs. the expected real variation) confirming the control loop stays stable -- no sustained oscillation or excessive overshoot. Not yet performed, see 13.5. If a burnout-configuration inertia is measured later, prefer a mid-burn average over the loaded value.

Mass and CG are NOT frozen -- both should vary over the burn per the motor thrust curve, interpolating from 743 g at ignition down to the burnout mass (743 g minus propellant mass, once supplied -- NOT down to the 656 g no-motor reference). Cheap to include and directly affects F=ma and static margin.

All values in 13.3 are tied to the CURRENT motor. A motor change moves CG (changing static margin -- currently 1.33 cal, but a heavier motor could push it below 1 cal), changes loaded inertia, and changes the thrust curve entirely. Action item: re-measure CG and re-run the loaded bifilar pendulum test with any new motor before trusting the sim.

## 13.5 Open Items

CP estimates disagree on the stability conclusion itself, not just the number: the CAD flat-plate estimate (36.1 cm) implies CP forward of CG (unstable); the OpenRocket/Barrowman estimate (47.8 cm) implies 1.33 cal (stable). Unresolved -- see 13.3/13.4.

Propellant mass and burnout mass are TODO placeholders pending the motor spec sheet (thrustcurve.org) -- needed for the mass model, 13.4

+-25% pitch-inertia robustness test REQUIRED before trusting the constant-inertia assumption -- not yet performed, see 13.4

CG (no motor, bench reference) not yet measured -- CG (loaded) now recorded, 13.3

Fusion's Steel/mass/inertia output describes the flat CP-estimation proxy, not the real rocket -- do not reuse it as an actual mass budget

Burnout-configuration pitch/yaw inertia -- optional under the constant-inertia assumption; can only be measured after a flight (single-use motor, needs a spent casing). Do NOT scale the loaded k^2 by a different mass -- must be measured.

Roll-axis inertia (loaded, and later burnout) -- not yet measured; hang nose-up, use the widest string spacing practical

If the motor changes: re-measure CG and re-run the loaded bifilar pendulum test with the new motor before trusting the sim -- see 13.4

# Appendix A: GPIO / Pin Assignment Table

Source: FlightComputer/src/config.h.

| Function | MCU Pin | Interface |
| --- | --- | --- |
| Pyro CH1 fire | 2 | GPIO, active HIGH |
| Pyro CH2 fire | 3 | GPIO, active HIGH |
| Pyro CH3 fire | 4 | GPIO, active HIGH |
| Pyro CH1 continuity | 40 | Analog input |
| Pyro CH2 continuity | 41 | Analog input |
| Pyro CH3 continuity | 39 | Analog input |
| Servo CH1 (S) | 36 | PWM |
| Servo CH2 (E) | 8 | PWM |
| Servo CH3 (N) | 7 | PWM |
| Servo CH4 (W) | 9 | PWM |
| LoRa radio | Serial1 (0/1) | UART 9600 |
| Camera | Serial7 (28/29) | UART 115200 |
| IMU (BNO085) | Wire (18/19) | I2C 0x4A |
| Barometer (BMP390) | Wire1 (17/16) | I2C 0x77 |
| GPS (ZOE-M8Q) | Wire1 (17/16) | I2C 0x42 |
| Power monitor (INA260) | Wire1 (17/16) | I2C 0x40 |
| High-g accel (ADXL375) | Wire2 (25/24) | I2C 0x1D |
| SD card | BUILTIN_SDCARD | SDIO 4-bit |
| GPS LED (D3) | 27 | GPIO, active HIGH |
| Altimeter LED (D4) | 5 | GPIO, active HIGH |
| 200G ACC LED (D5) | 13 | GPIO, active HIGH |
| IMU LED (D6) | 26 | GPIO, active HIGH |